# PowerVN8-AHDM

## Specification

### Purpose

PowerVN8-AHDM is an electronic module that is running the VideoNet® system and designed to:

  - receiving video signals from the digital high-definition cameras with analog signal transmission (AHDM) and the subsequent processing of video signals in the VideoNet® environment;

  - receiving the audio signal, its digitization and processing in the VideoNet® environment;

  - condition monitoring sensors;

  - control relay outputs.

### Тechnical features

Number of connected video cameras: 8

Number of linear audio inputs: 4 (8bit/8KHz)

Maximum frame rate: 200 fps

Number of connectors for connecting the MB-RIO 4/16 module: 1 (16 sensors и 4 relay outputs)

Connector type for connection to a computer: PCI Express х1

Type of connector for video and audio signals: DHS-15 F

Connector type for connecting MB-RIO 4/16 module: PLD-26

Power supply (power is supplied to the card via the PCI Express x1 connector): + 3,3 V

Consumption current: no more than 0.8 A

Overall dimensions: 130x100x25 mm

Weight: 90 г.

### Operating conditions

Ambient temperature: +5… +50°С

Relative air humidity at Т +20°С: not more than 80 %

Standard video signal: analog high definition system (AHD)

Standard audio signal: typical 0.5 V, maximum 1 V.

### Installation and connection

PowerVN8-AHDM video/audio capture board must be installed into the PCI Express х1/x4/х8/х16 slot of motherboard of your computer.

To connect video and audio sources to the PowerVN8-AHDM board, use the cable splitter included with the PowerVN8- AHDM board, see Fig. 2. The pin numbers of the BNC connectors on the splitter correspond to the camera numbers in the VideoNet® system.

Splitter is connected to the connector 1 of the PowerVN8-AHDM board (see Figure 1). It is recommended to monitor the status of the sensors and control the relay outputs via the MB-RIO 4/16 module. The MB-RIO 4/16 module is connected to the connector 2 of the PowerVN8-AHDM board (see Figure 1).

Drivers for the video/audio capture board are located on the VideoNet® installation DVD.

Attention!

The MB-RIO 4/16 module is purchased separately.

Using third-party drivers for this video/audio capture board is not allowed.

Note:

Before you start installing drivers, you need to make sure that there are no third-party drivers for the video/audio capture boards on the computer, otherwise an incorrect installation may be possible, resulting in the inoperability of the operating system.

Connector 1 - connector for video and audio signals

Connector 2 - connector for connecting the MB-RIO 4/16 module

Fig. 1.  PowerVN8-AHDM board connectors location

### Package contents

| - PowerVN8-AHDM board | 1 pc. |
| --- | --- |
| - Specification | 1 pc. |
| - Cable splitter | 1 pc. |
| - Packing (antistatic) | 1 pc. |

### Manufacturer’s warranty

Manufacturing Company SKYROS Corporation provides warranty of the product for 12 months from the date of sale. In case of the absence of a mark of release – warranty period is calculated from the date of manufacture of the product.

This warranty is void if the PowerVN8-HD board is out of order in case of:

• wrong connection;

• non-compliance with this guide;

• mechanical damage;

• disaster;

• signals that exceed standard values.

The manufacturer has the right to make changes in design of product, without affecting its basic technical parameters.

All questions of warranty and post-warranty repair, contact: 17, Remeslennaya st., St.Petersburg, Russia, tel. +7 (812) 448-10-02.

### Certificate of acceptance

The PowerVN8-AHDM board (serial number _______________) meets the specifications and is suitable for operation.

| Release date | __.__.__ | QCD stamp |
| --- | --- | --- |

### The mark of realization (sale)

| Date of sale |  |  |
| --- | --- | --- |
|  |  | (DD/MM/YY) |

| Seller |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | (signature) |  | (Full name) |

|  |  | stamp |  |  |
| --- | --- | --- | --- | --- |